main.c

#include <msp430.h>

#include "textconv.h"

/*

* Frequency Counter project - 002B

* Copyright by Zoltan Gomori

* License:

* Creative Commons Attribution-ShareAlike 3.0 Unported (CC BY-SA 3.0)

* main.c

*/

const char segments[] = {0x3F,0x06, 0x5B, 0x4F, 0x66, 0x6D, 0x7D, 0x07, 0x7F, 0x6F, 0x77, 0x7C, 0x39, 0x5E, 0x79, 0x71};

unsigned long prevccr; // Previous value of the capture register

unsigned long counter;

unsigned char bcd[8];

unsigned char pps1;

char pps1_count; // last DP blink - On intervall

int main(void) {

WDTCTL = WDTPW | WDTHOLD;	// Stop watchdog timer

// Set 16MHz clock

BCSCTL1 = CALBC1_16MHZ;

DCOCTL = CALDCO_16MHZ;

// initialize long counter

counter = 0;

// initialize GPIO (Optimalized)

P1DIR = 0xFC;

P1SEL = 0x03;

P1SEL2 = 0;

P2DIR = 0xFF;

P2SEL = 0;

P2SEL2 = 0;

// Initialize Timer_A0

// TACTL register

// BIT 9-8 - 00 - Source: TACLK (TASSEL_0)

// BIT 7-6 - 00 - Input divider: /1

// BIT 5-4 - 10 - Mode: Continuous (MC_2)

// BIT 1   -  1 - Enable TAIV interrupt (TAIE)

TA0CTL = TASSEL_0 + MC_2 + TAIE;

// Initialize compare block 0

// CAP = 1 - Use capture mode

// SCS = 0 - Asynchronous capture

// CM1 - Capture on rising edge

// CCIE - External input

TA0CCTL0 = CM1 + CAP + CCIE;

_EINT();	// Enable interrupts

pps1 = 0;

unsigned char i;

// infinite loop

while (1)

{

for(i=0;i<8;i++)

{

P2OUT = 0x00;

P1OUT &= 0x03;

P1OUT |= (i << 3) + 4;	// Select Digit + Enable digit decoder output

// (bcd[i] & 0x10) ^ 0x10) - Enable segment outputs if the suppress zero bit off

// (i ? 0 : 1) - Enable segment outputs if it is the LS digit

// (this overwrite the suppress when the total output is 0)

// segments[bcd[i] & 0x0F] - Segment display

// (i ? 0 : pps1) - Last DP blink

P2OUT = ((((bcd[i] & 0x10) ^ 0x10) | (i ? 0 : 1)) ? segments[bcd[i] & 0x0F] : 0) + (i ? 0 : pps1);

__delay_cycles(2000);

}

if(pps1_count > 0)

{

pps1_count--;

}

else

{

pps1 = 0; // switch of the last DP

}

}

}

// Timer capture interrupt

#pragma vector=TIMER0_A0_VECTOR

__interrupt void TACCR0_ISR(void)

{

// The execution of the overflow interrupt blocked from here

unsigned long freq;

// store the actual value of the counter

freq = counter;

// clear master counter

counter = 0;

// Clear interrupt

// The execution of the overflow interrupt allowed from here

TA0CCTL0 &= ~CCIFG;

// get the capture data

unsigned long ccrvalue;

ccrvalue = TA0CCR0;

// process data

//

//          || <--------------------------------------------------- freq ------------------------------------------------> ||

//  prevccr || 0x10000 - prevccr | 0x10000 overflow | 0x10000 overflow | ..... | 0x10000 overflow | 0x10000 overflow | ccr ||

freq += ccrvalue - prevccr;

// store current ccr for the next count

prevccr = ccrvalue;

// covert the binary frequency value to BCD for display

LongToBCD(8,bcd,freq,0);

pps1 = 0x80; // switch on the last DP

pps1_count = 100; // Keep Last DP on for 100 display cicles

}

// Timer_A3 Interrupt Vector (TA0IV) handler

#pragma vector=TIMER0_A1_VECTOR

__interrupt void TA0IV_ISR(void)

{

// after the counter overflow increment the counter with 65536

if(TAIV == 10)

{

counter += 0x10000;

}

}

textconv.h

/*

* Frequency Counter project - 002B

* Copyright by Zoltan Gomori

* License:

* Creative Commons Attribution-ShareAlike 3.0 Unported (CC BY-SA 3.0)

* textconv.h

*/

void LongToBCD(unsigned int len, unsigned char str[], unsigned long src, unsigned char leadingzero);

textconv.c

/*

* Frequency Counter project - 002B

* Copyright by Zoltan Gomori

* License:

* Creative Commons Attribution-ShareAlike 3.0 Unported (CC BY-SA 3.0)

* textconv.c

*/

void LongToBCD(unsigned int len, unsigned char str[], unsigned long src, unsigned char leadingzero)

{

unsigned long tmp;

unsigned char *i;

for(i=str; i < str+len;i++)

{

if(src > 0)

{

tmp = src % 10;

*i = (unsigned char)tmp;

src = src / 10;

}

else

{

*i = leadingzero ? 0 : 0x10;

}

}

return;

}